# Windows 11 as Internet Gateway for macOS Ventura

Created: 2026-01-06
Tags: #networking #windows #macos #ics #tutorial

## Overview

This guide provides a complete procedure for configuring a Windows 11 computer to act as an internet gateway for a macOS Ventura machine on the same local network. This setup uses Windows’ Internet Connection Sharing (ICS) feature.

Use Cases: - Sharing a single internet connection between multiple devices - Temporary network setup for home or small office - Testing network configurations - Situations where only one device has internet access

Important Notes: - ICS creates a separate subnet (192.168.137.x) for shared connections - The Windows PC must remain powered on for the Mac to access the internet - Performance depends on the Windows host’s capabilities - Not recommended for production environments or large networks

## Prerequisites

### Hardware Requirements

Windows 11 Computer (Gateway/Host): - Two network interfaces: - One connected to the internet (WiFi, Ethernet, or cellular) - One for sharing (typically Ethernet to connect to Mac) - Administrator privileges - Stable power supply

macOS Ventura Computer (Client): - Network interface (Ethernet or WiFi) - Network cable (if using wired connection)

### Network Setup Options

- Ethernet Connection (Recommended):

  - Connect Windows PC to internet via WiFi or primary Ethernet

  - Connect Mac to Windows PC via Ethernet cable (crossover or regular)

- WiFi Connection:

  - Windows PC connected to internet

  - Mac connects to Windows-created wireless network (Mobile Hotspot)

## Part 1: Windows 11 Configuration

### Step 1: Verify Network Adapters

- Press Win + R and type ncpa.cpl, then press Enter

- Identify your network adapters:

  - Internet Connection: The adapter currently connected to the internet (e.g., “WiFi” or “Ethernet”)

  - Sharing Connection: The adapter you’ll share through (e.g., “Ethernet 2” or secondary network card)

### Step 2: Enable ICS Service

- Press Win + R and type services.msc, then press Enter

- Scroll down to Internet Connection Sharing (ICS)

- Right-click on it and select Properties

- Set the Startup type to Automatic

- Click Start if the service isn’t running

- Click Apply → OK

### Step 3: Configure Internet Connection Sharing

- Press Win + R and type ncpa.cpl, then press Enter

- Right-click on your Internet Connection adapter (the one with internet access)

- Select Properties

- Click the Sharing tab

- Check the box: “Allow other network users to connect through this computer’s Internet connection”

- From the dropdown menu labeled “Home networking connection:”, select your Sharing Connection adapter (the one connected to your Mac)

- (Optional) Check “Allow other network users to control or disable the shared Internet connection”

- Click OK

Expected Result: Windows will display a notification that the sharing adapter will be set to IP address 192.168.137.1

### Step 4: Verify Sharing Adapter Configuration

- Press Win + R and type ncpa.cpl, then press Enter

- Right-click on your Sharing Connection adapter

- Select Properties → Internet Protocol Version 4 (TCP/IPv4) → Properties

- Verify it’s set to:

  - IP address: 192.168.137.1

  - Subnet mask: 255.255.255.0

- Click OK to close all dialogs

### Step 5: Configure Windows Firewall (If Necessary)

- Press Win + I to open Settings

- Navigate to Privacy & Security → Windows Security → Firewall & network protection

- Ensure the firewall allows ICS traffic

- If connection issues persist, temporarily disable the firewall to test (remember to re-enable it)

## Part 2: macOS Ventura Configuration

### Method A: Automatic Configuration (DHCP)

Note: Some users report that ICS DHCP doesn’t always work reliably. If this method fails, use Method B.

- Click the Apple menu → System Settings

- Click Network in the sidebar

- Select your network interface (Ethernet or WiFi)

- Click Details

- Click TCP/IP in the left sidebar

- Verify Configure IPv4 is set to Using DHCP

- Click OK

- Wait for the Mac to obtain an IP address automatically (192.168.137.x range)

Test Connection:

ping 8.8.8.8

If successful, you should see replies. If this method doesn’t work, proceed to Method B.

### Method B: Manual Configuration (Static IP)

This method is more reliable when ICS DHCP service isn’t working properly.

- Click the Apple menu → System Settings

- Click Network in the sidebar

- Select your network interface (Ethernet or WiFi connected to Windows PC)

- Click Details

- Click TCP/IP in the left sidebar

- Click the Configure IPv4 dropdown and select Manually

- Enter the following settings:

  - IPv4 Address: 192.168.137.10 (or any number from 2-254 except 1)

  - Subnet Mask: 255.255.255.0

  - Router: 192.168.137.1 (this is the Windows PC)

- Click DNS in the left sidebar

- Click the + button and add DNS servers:

  - Primary: 8.8.8.8 (Google DNS)

  - Secondary: 8.8.4.4 (Google DNS alternate)

  - Or use: 1.1.1.1 and 1.0.0.1 (Cloudflare DNS)

- Click OK

Test Connection:

# Test gateway connectivity
ping 192.168.137.1

# Test internet connectivity
ping 8.8.8.8

# Test DNS resolution
ping google.com

## Part 3: Troubleshooting

### Issue 1: Mac Not Getting IP Address (DHCP Failure)

Symptoms: - Mac shows “Self-Assigned IP” or 169.254.x.x address - No internet connectivity

Solution: - Use Method B (Manual Configuration) instead - On Windows, verify the ICS service is running (services.msc) - Restart both the Windows PC and Mac - Disable and re-enable ICS on Windows

### Issue 2: Mac Gets IP But No Internet

Symptoms: - Mac has 192.168.137.x address - Can ping 192.168.137.1 (Windows gateway) - Cannot reach internet sites

Solutions:

On Windows: 1. Check Windows Firewall settings 2. Verify the internet connection is active on Windows 3. Disable ICS, restart, then re-enable ICS 4. Run Command Prompt as Administrator: cmd    netsh winsock reset    netsh int ip reset    ipconfig /release    ipconfig /renew    ipconfig /flushdns 5. Restart Windows PC

On Mac: 1. Verify DNS settings are correct (see Method B, step 9) 2. Clear DNS cache: bash    sudo dscacheutil -flushcache    sudo killall -HUP mDNSResponder 3. Try different DNS servers

### Issue 3: Intermittent Connection

Symptoms: - Connection works then stops - Slow or unstable performance

Solutions: - Ensure Windows PC isn’t going to sleep or hibernating - Disable power saving on network adapters: 1. Win + R → devmgmt.msc 2. Expand Network adapters 3. Right-click each adapter → Properties 4. Power Management tab 5. Uncheck “Allow the computer to turn off this device to save power” - Check for Windows updates - Update network adapter drivers on Windows

### Issue 4: “Network is Inactive” Error on Mac

Symptoms: - macOS shows network as inactive despite being connected - Can’t apply network settings

Solutions: 1. Disconnect and reconnect the network cable 2. Turn WiFi off and on (if using WiFi) 3. Create a new network location: - System Settings → Network - Click the ⋯ (three dots) → Set Service Order - Click + to add a new location - Configure the new location with Manual settings 4. Reset network settings on Mac: bash    sudo ifconfig en0 down    sudo ifconfig en0 up

### Issue 5: Windows ICS Service Won’t Start

Symptoms: - Error when enabling ICS - Service fails to start

Solutions: 1. Check for conflicting services (VPN, WSL, virtual machines) 2. Verify both network adapters are enabled and functioning 3. Run Command Prompt as Administrator: cmd    sc config SharedAccess start= auto    net start SharedAccess 4. Check Event Viewer for specific error messages: - Win + X → Event Viewer - Navigate to Windows Logs → System - Look for SharedAccess errors

### Issue 6: Security Software Blocking Connection

Symptoms: - Connection fails despite correct configuration - Firewall warnings

Solutions: - Temporarily disable third-party antivirus/firewall to test - Add exceptions for the 192.168.137.x subnet - Ensure ports 67 and 68 (UDP) are open for DHCP

## Part 4: Testing and Verification

### On Windows (Gateway)

- Open Command Prompt:

- ipconfig /all

- Verify the sharing adapter shows:

  - IP: 192.168.137.1

  - Subnet: 255.255.255.0

- Check ICS status:

- sc query SharedAccess

- Should show STATE: RUNNING

### On macOS (Client)

- Open Terminal and run:

- ifconfig

- Look for your interface (en0, en1, etc.) and verify it has a 192.168.137.x address

- Check routing:

- netstat -nr | grep default

- Should show 192.168.137.1 as the default gateway

- Test connectivity:

- # Test gateway
ping -c 4 192.168.137.1

# Test internet
ping -c 4 8.8.8.8

# Test DNS
ping -c 4 google.com

# Check trace route
traceroute google.com

- Test actual browsing:

  - Open Safari or another browser

  - Navigate to various websites

  - Test HTTPS sites specifically

## Part 5: Performance Optimization

### Optimize Windows Host

- Disable unnecessary network features:

  - Network adapter properties → Disable IPv6 if not needed

  - Disable network discovery if not required

- Update network drivers:

  - Win + X → Device Manager

  - Update all network adapter drivers

- Adjust power settings:

  - Control Panel → Power Options → Change plan settings

  - Set to High Performance when acting as gateway

### Optimize macOS Client

- Disable unused network services:

- # List all network services
networksetup -listallnetworkservices

# Disable unused services (example)
sudo networksetup -setnetworkserviceenabled "Bluetooth PAN" off

- Set MTU if needed:

- networksetup -setMTU Ethernet 1500

## Part 6: Alternative Methods

### Using Mobile Hotspot (Windows 11)

If you prefer wireless sharing:

- Press Win + I → Network & Internet

- Click Mobile hotspot

- Turn on Share my Internet connection with other devices

- Select the internet connection to share from dropdown

- Choose WiFi as sharing method

- Note the Network name and Password

- Configure security settings as needed

On macOS: - Connect to the WiFi network created by Windows - Should work automatically with DHCP

### Using Third-Party Software

Alternative tools if ICS doesn’t meet your needs: - Connectify Hotspot (Windows) - Virtual Router (Windows, open-source) - Physical router/switch (hardware solution)

## Part 7: Security Considerations

### Important Security Notes

- All traffic routes through Windows PC:

  - The Windows host can potentially monitor all network traffic

  - Not recommended for sensitive corporate networks

- Basic NAT protection only:

  - ICS provides basic Network Address Translation

  - No advanced firewall features

  - No intrusion detection

- Recommendations:

  - Use strong passwords on Windows account

  - Keep Windows fully updated

  - Enable Windows Defender/antivirus

  - Use VPN on macOS for sensitive traffic

  - Don’t use for production/enterprise environments

- Network isolation:

  - ICS creates a separate subnet (192.168.137.x)

  - Mac cannot directly access devices on Windows’ main network

  - This provides some isolation but is not a security feature

## Part 8: Removing ICS Configuration

### To Disable ICS on Windows

- Press Win + R and type ncpa.cpl

- Right-click the internet connection adapter

- Select Properties → Sharing tab

- Uncheck “Allow other network users to connect through this computer’s Internet connection”

- Click OK

- The sharing adapter will revert to DHCP or its previous configuration

### To Reset macOS Network Settings

- System Settings → Network

- Select the interface used for ICS

- Click Details

- Change Configure IPv4 back to Using DHCP

- Remove custom DNS servers if added

- Click OK

## Quick Reference: Network Settings Summary

### Windows Gateway Configuration

Internet Adapter: Connected to internet (no changes needed)
Sharing Adapter:
  - IP Address: 192.168.137.1
  - Subnet Mask: 255.255.255.0
  - DHCP Enabled: Yes (automatic via ICS)
ICS Service: Running (Automatic startup)

### macOS Client Configuration (Manual Method)

IP Address: 192.168.137.10 (or any 2-254)
Subnet Mask: 255.255.255.0
Router/Gateway: 192.168.137.1
DNS Servers: 8.8.8.8, 8.8.4.4

## Common Commands Reference

### Windows Commands

# View network configuration
ipconfig /all

# Release and renew IP
ipconfig /release
ipconfig /renew

# Flush DNS cache
ipconfig /flushdns

# Reset network stack
netsh winsock reset
netsh int ip reset

# Check ICS service status
sc query SharedAccess

# Start ICS service
net start SharedAccess

# View network connections
ncpa.cpl

### macOS Commands

# View network configuration
ifconfig

# View routing table
netstat -nr

# List network services
networksetup -listallnetworkservices

# Renew DHCP lease
sudo ipconfig set en0 DHCP

# Set manual IP (en0 is typically Ethernet)
sudo networksetup -setmanual "Ethernet" 192.168.137.10 255.255.255.0 192.168.137.1

# Set DNS servers
sudo networksetup -setdnsservers "Ethernet" 8.8.8.8 8.8.4.4

# Flush DNS cache
sudo dscacheutil -flushcache
sudo killall -HUP mDNSResponder

# Test connectivity
ping 192.168.137.1      # Test gateway
ping 8.8.8.8            # Test internet
ping google.com         # Test DNS
traceroute google.com   # Trace route to internet

## Resources and References

### Official Documentation

- Microsoft: Internet Connection Sharing Documentation

- Apple: macOS Network Settings

### Key Concepts

- ICS (Internet Connection Sharing): Windows feature that enables sharing an internet connection

- NAT (Network Address Translation): Technology that allows multiple devices to share a single public IP

- DHCP (Dynamic Host Configuration Protocol): Automatic IP address assignment

- Gateway: Device that routes traffic between networks (Windows PC in this setup)

- Static IP: Manually configured IP address that doesn’t change

### Limitations to Be Aware Of

- Performance depends on Windows PC hardware

- Windows PC must remain powered on

- Single point of failure (if Windows crashes, Mac loses internet)

- Limited to small networks (home/small office use)

- No advanced network features (QoS, VLANs, etc.)

- ICS DHCP server sometimes unreliable (hence manual configuration option)

## Notes and Tips

- For best results: Use Method B (Manual Configuration) on macOS as it’s more reliable

- Connection stability: Keep Windows PC plugged into power and disable sleep/hibernation

- Testing: Always test with ping before assuming internet access is working

- DNS issues: If websites don’t load but ping works, it’s likely a DNS problem

- Corporate networks: Check with IT before using ICS; many organizations block it

- VPN usage: If Windows is using VPN, all Mac traffic will route through the VPN

- Backup connection: Keep alternative internet access method available during setup

## Changelog

- 2026-01-06: Initial documentation created

- Covers Windows 11 (Build 22000+) and macOS Ventura (13.x)

Last Updated: January 6, 2026
Status: Active Guide
Difficulty Level: Intermediate